Communication Protocol

ClairWav Radar Detectors

V2.1

(For Loranet only)

Content:

1.	Communication Protocol for Models without Camera	4

1.1 Communication Conventions	4

1.2 Data Type Definitions	6

1.3 Data Packet Format	7

1.3.1 Data Frame Format	7

1.3.2 Data Length	9

1.3.3 Device ID	10

1.3.4 Data timestamp	11

1.3.5 Target trajectory data (0x01：ObjectData)	13

1.3.6 Traffic flow statistics data (0x03：TrafficData)	17

2.	Communication Protocol for Video Integrated Radar Models	21

2.1 Communication Protocol	21

2.2 Data Packet Format	21

2.2.1 Data Frame Format	22

2.2.2 Target Data	23

2.2.3 Regional Statistical Data (0x02: RegionData)	28

2.2.4 Traffic Flow Statistical Data (0x03：TrafficData)	29

2.2.5 Lane Status Data (0x04: LaneStatusData)	33

2.2.6 Real-time passing vehicle data (0x05: PassData)	35

- Communication Protocol for Models without Camera

Note: The following communication protocol is optimized for ClairWav-T24/T80/U24 models. It is specifically designed for applications that do not require additional video-based content, such as license plate recognition or detailed vehicle classifications.

1.1 Communication Conventions

1.To ensure the real-time and reliable transmission of data, the communication uses the standard TCP protocol. The radar acts as a TCP client, and each radar establishes a TCP connection with the MEC.

2. Data is transmitted using network byte order.

3. The definition of the radar data reference frame is illustrated in the following diagram:

The radar position is taken as the origin of the coordinate system, with the direction of the radar's normal defined as the Y-axis and the direction perpendicular to the normal defined as the X-axis. All coordinate data, velocity, and acceleration in the protocol are based on this standard.

1.2 Data Type Definitions

| Data Type | Abbreviation | Byte |
| --- | --- | --- |
| char | T | 1 |
| unsigned char | UT | 1 |
| short | S | 2 |
| unsigned short | US | 2 |
| int | L | 4 |
| unsigned int | UL | 4 |
| float | float | 4 |
| double | double | 8 |
| string | C | NotFixed |

1.3 Data Packet Format

The data packet consists of a header, data body, and footer.

The reporting interval of target trajectory data: 100ms.

The reporting interval of traffic flow statistics data: 1 minute.

Notes:

1. If the server does not receive radar data for 20 seconds, the TCP connection will be terminated. When the radar sends data again, it must initiate a new connection request.

2. If there are no targets, the radar should still periodically report target trajectory data according to the data frame format, but the "target data" field will be empty.

1.3.1 Data Frame Format

| NO. | Name | Bytes | Description |
| --- | --- | --- | --- |
| 1 | Header | 4 | 55 AA 55 BB |
| 2 | Data length | 2 | The total length of bytes from index 2 to index 7 |
| 3 | Data frame type | 1 | 0x01：ObjectData 0x03：TrafficData |
| 4 | Checksum |  | Indexes 5 to 7 are involved in the checksum calculation. Checksum algorithm: The checksum is calculated by summing up the bytes in the data storage area and taking the low 8 bits of the sum. Code: // unsigned char* data Data buffer pointer // int start The position where checksum calculation starts // int len The number of bytes involved in checksum calculation unsigned char checkSum(unsigned char* data, int start, int len ) { int sum=0; for(int i=start; i<len; i++) { sum+=*(data+i); } unsigned char ret=sum&0xFF; return ret; } |
| 5 | Device ID | 20 | Taken from the device installation parameters |
| 6 | Data timestamp | 8 | The time of data transmission (accurate to milliseconds) |
| 7 | Target data | N*Length | Collection of target data N <= 256, indicating the number of targets; Length: (1) When the data frame type is ObjectData, the value is 36; (2) When the data frame type is TrafficData, the value is 22. |
| 8 | Footer | 4 | 55 CC 55 DD |

1.3.2 Data Length

| NO. | bit15 | bit14 | bit13 | bit12 | bit11 | bit10 | bit9 | bit8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Function | length[15:8] high 8 bits |  |  |  |  |  |  |  |
| NO. | bit7 | bit6 | bit5 | bit4 | bit3 | bit2 | bit1 | bit0 |
| Function | length[7:0] low 8 bits |  |  |  |  |  |  |  |

1.3.3 Device ID

| NO. | Byte1 | Byte2 | Byte3 | Byte4 | Byte5 | Byte6 | Byte7 | Byte8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Function | DEV_ID |  |  |  |  |  |  |  |
| NO. | Byte9 | Byte10 | Byte11 | Byte12 | Byte13 | Byte14 | Byte15 | Byte16 |
| Function | DEV_ID |  |  |  |  |  |  |  |
| NO. | Byte17 | Byte18 | Byte19 | Byte20 |  |  |  |  |
| Function | DEV_ID |  |  |  |  |  |  |  |

The device ID is a maximum of 20 bytes, taken from the device installation parameters. Each byte represents a character's ASIC code.

For example,

Device ID is: 0000001

DEV_ID: 0x30 0x30 0x30 0x30 0x30 0x30 0x31

| NO. | Byte1 | Byte2 | Byte3 | Byte4 | Byte5 | Byte6 | Byte7 | Byte8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Function | 0x30 | 0x30 | 0x30 | 0x30 | 0x30 | 0x30 | 0x31 | 0x00 |
| NO. | Byte9 | Byte10 | Byte11 | Byte12 | Byte13 | Byte14 | Byte15 | Byte16 |
| Function | 0x00 | 0x00 | 0x00 | 0x00 | 0x00 | 0x00 | 0x00 | 0x00 |
| NO. | Byte17 | Byte18 | Byte19 | Byte20 |  |  |  |  |
| Function | 0x00 | 0x00 | 0x00 | 0x00 |  |  |  |  |

1.3.4 Data timestamp

| NO. | Byte1 | Byte2 | Byte3 | Byte4 | Byte5 | Byte6 | Byte7 | Byte8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Function | year | month | day | hour | min | sec | ms |  |

The data timestamp represents the time when radar data was updated, accurate to milliseconds, and is represented using 8 bytes.

1) year data bit description

| NO. | bit7 | bit6 | bit5 | bit4 | bit3 | bit2 | bit1 | bit0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Function | Data 0 to 255 corresponds to the years 2000 to 2255. |  |  |  |  |  |  |  |

2) month data bit description

| NO. | bit7 | bit6 | bit5 | bit4 | bit3 | bit2 | bit1 | bit0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Function | Reserved | Reserved | Reserved | Reserved | month[3:0]: 1-12 month |  |  |  |

3) day data bit description

| NO. | bit7 | bit6 | bit5 | bit4 | bit3 | bit2 | bit1 | bit0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Function | Reserved | Reserved | Reserved | day[4:0]: 1-31 day |  |  |  |  |

4) hour data bit description

| NO. | bit7 | bit6 | bit5 | bit4 | bit3 | bit2 | bit1 | bit0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Function | Reserved | Reserved | Reserved | hour[4:0]: 0-23 hour |  |  |  |  |

5) min data bit description

| NO. | bit7 | bit6 | bit5 | bit4 | bit3 | bit2 | bit1 | bit0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Function | Reserved | Reserved | min[5:0]: 0-59 min |  |  |  |  |  |

6) sec data bit description

| NO. | bit7 | bit6 | bit5 | bit4 | bit3 | bit2 | bit1 | bit0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Function | Reserved | Reserved | sec[5:0]: 0-59 sec |  |  |  |  |  |

7) ms data bit description

| NO. | bit15 | bit14 | bit13 | bit12 | bit11 | bit10 | bit9 | bit9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Function | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | ms[9:9] |  |
| NO. | bit7 | bit6 | bit5 | bit4 | bit3 | bit2 | bit1 | bit0 |
| Function | ms[7:0] |  |  |  |  |  |  |  |

Note: The range of values for ms[0:9] is from 0 to 999.

1.3.5 Target trajectory data (0x01：ObjectData)

| NO. | Name | Type | Bytes | Description |
| --- | --- | --- | --- | --- |
| 1 | Target number | US | 2 | Loop ID allocation, range: 0 to 65535 |
| 2 | Lane to which the target belongs | UT | 1 | The target coordinates are located in the specific lane number set by the user. Radar monitors specific lane numbers using the following rules: For single-lane roads: - Upstream: 01 - Downstream: 03 For roads with more than two lanes: - Upstream (closer to the radar): Lane numbers are consecutively assigned from inner (innermost lane) to outer (outermost lane), starting from 11. - Downstream (away from the radar): Lane numbers are consecutively assigned from inner (innermost lane) to outer (outermost lane), starting from 31. The arrangement rule for lane numbers is as follows: first upstream, then downstream. Within the same direction of travel, inner lanes are numbered before outer lanes. If the target is not within a lane, the value is 0xFF. An invalid value is represented by 0x00. |
| 3 | Target type | UT | 1 | 0X00—not used 0X01—non-motor 0X02—car 0X03—bus |
| 4 | Target length | UT | 1 | Vehicle body length, unit: meters Range: 0 to 25.5 meters Value 0 indicates invalid data 1 decimal place is reserved, with a resolution of 0.1 meters |
| 5 | Target width | UT | 1 | Vehicle body width, unit: meters Range: 0 to 25.5 meters Value 0 indicates invalid data 1 decimal place is reserved, with a resolution of 0.1 meters |
| 6 | Target height | UT | 1 | Vehicle body height, unit: meters Range: 0 to 25.5 meters Value 0 indicates invalid data 1 decimal place is reserved, with a resolution of 0.1 meters |
| 7 | Reserved | UT | 3 | 0xFFFFFF |
| 8 | Target yaw angle | US | 2 | The current heading direction of the target in the geographical sense. The value range is 0 to 3599, representing 0 to 359.9 degrees. North is 0 degrees, and calculations proceed clockwise. The resolution is 0.1 degrees. |
| 9 | X-coordinate | S | 2 | X-coordinate of the target object relative to the radar coordinate system, unit: meters. Resolution: 0.01 meters. |
| 10 | Y-coordinate | US | 2 | Y-coordinate of the target object relative to the radar coordinate system, unit: meters. Resolution: 0.01 meters. |
| 11 | X-axis velocity | S | 2 | X-axis velocity of the detected object relative to the radar coordinate system, unit: meters per second. |
| 12 | Y-axis velocity | S | 2 | Y-axis velocity of the detected object relative to the radar coordinate system, unit: meters per second. Resolution: 0.01 meters per second. |
| 13 | Velocity | S | 2 | Converted velocity of the detected object, unit: kilometers per hour. Resolution: 0.01 kilometers per hour. Positive value indicates moving away from the radar; negative value indicates moving closer to the radar. |
| 14 | Acceleration of the target in the direction of motion | S | 2 | Acceleration of the detected object in the direction of motion, unit: meters per second squared. Range: (-15, 15). Resolution: 0.01 meters per second squared. |
| 15 | X-axis acceleration | S | 2 | X-axis acceleration of the detected object relative to the radar coordinate system, unit: meters per second squared. Range: (-15, 15). Resolution: 0.01 meters per second squared. |
| 16 | Y-axis acceleration | S | 2 | Y-axis acceleration of the detected object relative to the radar coordinate system, unit: meters per second squared. Range: (-15, 15). Resolution: 0.01 meters per second squared. |
| 17 | Longitude of the target coordinate point | L | 4 | Longitude of the target coordinate point is defined by longitude values, where positive values represent East longitude and negative values represent West longitude. Range: (-1799999999 to 1800000001) Unit: 1/10 microdegree, resolution: 1e-7° Providing a range of -1800000000 to 1800000000 to cover positive and negative 180 degrees. |
| 18 | Latitude of the target coordinate point | L | 4 | Latitude of the target coordinate point is defined by latitude values, where positive values represent North latitude and negative values represent South latitude. Range: (-900000000 to 900000001) Unit: 1/10 microdegree, resolution: 1e-7° Providing a range of positive and negative 90 degrees. |

1.3.6 Traffic flow statistics data (0x03：TrafficData)

| NO. | Name | Type | Bytes | Description |
| --- | --- | --- | --- | --- |
| 1 | Statistical period | US | 2 | The latest unit: 1 second |
| 2 | Lane to which the target belongs | US | 2 | The target coordinates are located in the specific lane number set by the user. The radar monitors specific lane numbers using the following rules: For single-lane roads: - Upstream: 01 - Downstream: 03 For roads with more than two lanes: - Upstream (closer to the radar): Lane numbers are consecutively assigned from inner (innermost lane) to outer (outermost lane), starting from 11. - Downstream (away from the radar): Lane numbers are consecutively assigned from inner (innermost lane) to outer (outermost lane), starting from 31. The arrangement rule for lane numbers is as follows: first upstream, then downstream. Within the same direction of travel, inner lanes are numbered before outer lanes. For roundabout lane numbers, they are consecutively assigned from inner to outer, starting from 51. For channelized lane numbers, they are consecutively assigned from inner to outer, starting from 71. For exit lane numbers, they are consecutively assigned from inner to outer, starting from 91. If the target is not within a lane, the value is 0xFF. An invalid value is represented by 0x00. |
| 3 | Monitoring location | US | 2 | Distance in the Y direction relative to the stop line, with a resolution of 0.1 meters. |
| 4 | Minor traffic flow | US | 2 | Vehicles |
| 5 | Major traffic flow | US | 2 | Vehicles |
| 6 | Heavy-duty vehicle flow | US | 2 | Vehicles |
| 7 | Total flow | US | 2 | Vehicles |
| 8 | Average vehicle speed | US | 2 | Resolution: 0.1 kilometers per hour |
| 9 | Headway | US | 2 | seconds |
| 10 | Virtual loop occupancy time | US | 2 | Percentage, resolution: 0.1 |
| 11 | Maximum queue length | US | 2 | Resolution: 0.1 meters PS: Queue length starts from the stop line |

- Communication Protocol for Video Integrated Radar Models

Note: The following communication protocol is for video integrated radar models including ClairWav-T24SC/T24LC/T80LC.

2.1 Communication Protocol

1. To ensure real-time and reliable data transmission, the communication protocol uses the standard TCP protocol. The radar functions as either a TCP client or server, with each radar establishing a single TCP connection with the receiving device.

2. Data is transmitted in network byte order.

2.2 Data Packet Format

Each data packet consists of a header, a data body, and a footer.

1. If the server does not receive radar data for 20 seconds, it will disconnect the TCP connection. When the radar sends data again, it must re-initiate the connection request.

2. Even if no targets are detected, the radar should periodically report target trajectory data according to the data frame format, with the "Target Data" field left empty.

2.2.1 Data Frame Format

| NO, | Name | Byte | Description |
| --- | --- | --- | --- |
| 1 | Header | 4 | 55 AA 55 BB |
| 2 | Data Frame Types | 1 | 0x01：ObjectData 0x02：RegionData 0x03：TrafficData 0x04：LaneStatusData 0x05：PassData |
| 3 | Checksum | 1 | Sequence numbers 5 to 7 participate in the checksum. |
| 4 | Reserved | 2 |  |
| 5 | Device ID | 20 | Obtained from device installation parameters. |
| 6 | Data Timestamp | 8 | UTC milliseconds since 1970. |
| 7 | Number of Data Entries | 2 | N |
| 8 | Target Data | N*Length |  |
| 9 | Packet Footer | 4 | 55 CC 55 DD |

2.2.2 Target Data

| NO, | Name | Byte | Description |
| --- | --- | --- | --- |
| 1 | Target ID | 10 | The first 8 bytes represent the timestamp when the target was generated, and the last 2 bytes are the cyclic ID. Cyclic ID allocation range: 0 to 65535. |
| 2 | Target Lane | 1 | The target's coordinate point is located within the specific lane number set by the user. Radar monitoring specific lane numbering rules are as follows: - Single Lane Code Rules: - Upstream: 01 - Downstream: 03 - Multi-Lane Road Code Rules - Upstream (closest to the radar): Lanes are numbered sequentially from the inner lane (inner lane) to the outer lane (outer lane) as 11, 12, 13, etc. - Downstream (farthest from the radar): Lanes are numbered sequentially from the inner lane (inner lane) to the outer lane (outer lane) as 31, 32, 33, etc. Lane number arrangement rules: upstream lanes are listed first, followed by downstream lanes. Within the same direction of travel, inner lanes are listed before outer lanes. - If the target is not within any lane, the value is set to 0xFF. - Invalid values are set to 0x00. |
| 3 | Target Type | 1 | 0X00—other 0X01—bicycle 0X02—motorcycle 0X03—tricycle 0X04—bus 0X05—van 0X06—car 0X07—SUV 0X08—large truck 0X09—medium truck 0X0A—light truck 0X0B—dangerous goods cars 0X0C—engineering vehicles 0X0D—person 0X0E—medium bus |
| 4 | Color | 1 | 0X00—other 0X01—white 0X02—gray 0X03—yellow 0X04—pink 0X05—red 0X06—purple 0X07—green 0X08—blue 0X09—brown 0X0A—black |
| 5 | Plate number | 12 |  |
| 6 | X Coordinate | 2 | The unit is in meters, with a resolution of 0.1 meters. |
| 7 | Y Coordinate | 2 | The unit is in meters, with a resolution of 0.1 meters. |
| 8 | Speed | 2 | The unit is in meters per second (m/s), with a resolution of 0.1 m/s. Positive values indicate movement away from the radar, while negative values indicate movement towards the radar. |
| 9 | X-axis in the image. | 2 | Unit: Pixels Invalid Value: -1 |
| 10 | Y-axis in the image. | 2 | Unit: Pixels Invalid Value: -1 |
| 11 | Target Pixel Height | 2 | Unit: Pixels Invalid Value: -1 |
| 12 | Target Pixel Width | 2 | Unit: Pixels Invalid Value: -1 |
| 13 | Target Azimuth Angle | 2 | Resolution: 0.1 degrees. |
| 14 | Vehicle Length | 1 | Unit: meters (m) Resolution: 0.1 meters. |
| 15 | Vehicle Width | 1 | Unit: meters (m) Resolution: 0.1 meters. |
| 16 | Vehicle Height | 1 | Unit: meters (m) Resolution: 0.1 meters. |
| 17 | Parking Status | 1 | 1. Parked. 0. Not Parked. |
| 18 | X Direction Speed | 2 | Unit: meters per second (m/s) Resolution: 0.1 m/s. |
| 19 | Y Direction Speed | 2 | Unit: meters per second (m/s) Resolution: 0.1 m/s. |
| 20 | Target Motion Direction Acceleration | 2 | The acceleration of the target's motion direction is measured in meters per second squared (m/s²), with a value range of (-15, 15) and a resolution of 0.01 m/s². |
| 21 | X Axis Acceleration | 2 | The X-axis acceleration of the target relative to the radar coordinate system is measured in meters per second squared (m/s²), with a value range of (-15, 15) and a resolution of 0.01 m/s². |
| 22 | Y Axis Acceleration | 2 | The Y-axis acceleration of the target relative to the radar coordinate system is measured in meters per second squared (m/s²), with a value range of (-15, 15) and a resolution of 0.01 m/s². |
| 23 | Longitude | 4 | Unit: 1/10 microdegree Resolution: 1e-7° |
| 24 | Latitude | 4 | Unit: 1/10 microdegree Resolution: 1e-7° |
| 25 | Reserved | 2 |  |

2.2.3 Regional Statistical Data (0x02: RegionData)

| NO. | Name | Type | Bytes | Description |
| --- | --- | --- | --- | --- |
| 1 | Statistical Period | US | 2 | Unit: Seconds |
| 2 | Direction of Belonging | US | 2 | 1. North 2. East 3. South 4. West 5. Northeast 6. Southeast 7. Northwest 8. Southwest |
| 3 | Left Turn Percentage | US | 2 | Percentage, with a resolution of 0.1. |
| 4 | Straight Percentage | US | 2 | Percentage, with a resolution of 0.1. |
| 5 | Right Turn Percentage | US | 2 | Percentage, with a resolution of 0.1. |
| 6 | Reserved | US | 2 |  |

2.2.4 Traffic Flow Statistical Data (0x03：TrafficData)

| NO. | Name | Bytes | Description |
| --- | --- | --- | --- |
| 1 | Statistical Period | 2 | Latest Unit: 1 Second |
| 2 | Target Lane | 2 | The target's coordinate point is located within the specific lane number set by the user. Radar monitoring specific lane numbering rules are as follows: - Single Lane Code Rules: - Upstream: 01 - Downstream: 03 - Multi-Lane Road Code Rules: - Upstream (closest to the radar): Lanes are numbered sequentially from the inner lane (inner lane) to the outer lane (outer lane) as 11, 12, 13, etc. - Downstream (farthest from the radar): Lanes are numbered sequentially from the inner lane (inner lane) to the outer lane (outer lane) as 31, 32, 33, etc. Lane number arrangement rules: upstream lanes are listed first, followed by downstream lanes. Within the same direction of travel, inner lanes are listed before outer lanes. - Roundabout Lane Numbering: Lanes are numbered sequentially from the inner lane to the outer lane as 51, 52, 53, etc. - Channelized Lane Numbering: Lanes are numbered sequentially from the inner lane to the outer lane as 71, 72, 73, etc. - Exit Lane Numbering: Lanes are numbered sequentially from the inner lane to the outer lane as 91, 92, 93, etc. - If the target is not within any lane, the value is set to 0xFF. - Invalid values are set to 0x00. |
| 3 | Monitoring Location | 2 | Y-direction distance relative to the stop line, with a resolution of 0.1 meters. |
| 4 | Bicycle Flow | 2 | 0x01 — Bicycle Unit: Units (vehicles) |
| 5 | Motorcycle Flow | 2 | 0x02 — Motorcycle Unit: Units (vehicles) |
| 6 | Tricycle Flow | 2 | 0X03 — Tricycle Unit: vehicles |
| 7 | Bus Flow | 2 | 0X04—Bus Unit: Vehicles. |
| 8 | Van Flow | 2 | 0X05 – Van Unit: Vehicles |
| 9 | Car Flow | 2 | 0X06—Car 0X00—Other Unit: Vehicles. |
| 10 | SUV Flow | 2 | 0X07 – SUV Unit: Vehicles |
| 11 | Large Truck Flow | 2 | 0X08 – Large Truck Unit: Vehicles |
| 12 | Medium Truck Flow | 2 | 0X09 – Medium Truck Unit: Vehicles |
| 13 | Light Truck Flow | 2 | 0X0A – Light Truck Unit: Vehicles |
| 14 | Dangerous Goods Car Flow | 2 | 0X0B – Dangerous Goods Cars Unit: Vehicles |
| 15 | Engineering Vehicles Flow | 2 | 0X0C – Engineering Vehicles Unit: Vehicles |
| 16 | Pedestrian Flow | 2 | 0X0D – Pedestrians Unit: Person |
| 17 | Total Flow | 2 | Unit: Vehicles |
| 18 | Average vehicle speed | 2 | Resolution 0.1 km/h |
| 19 | Headway time | 2 | Seconds |
| 20 | Virtual loop time occupancy rate | 2 | Percentage, resolution 0.1 |
| 21 | Maximum queue length | 2 | Resolution 0.1 m PS: queue length starts from the stop line. |
| 22 | Lane space occupancy rate | 2 | Percentage, resolution 0.1 |
| 23 | Vehicle spacing | 2 | Resolution 0.1 |
| 24 | Traffic density | 2 | Resolution 0.1 |
| 25 | Reserved | 2 |  |

2.2.5 Lane Status Data (0x04: LaneStatusData)

| NO. | Name | Type | Bytes | Description |
| --- | --- | --- | --- | --- |
| 1 | Lane NO. | US | 2 | Road code rules for lanes: For upward traffic (toward the radar), lanes are numbered consecutively from the inner (inner lane) to the outer (outer lane) as 11, 12, 13, etc. For downward traffic (away from the radar), lanes are also numbered consecutively from the inner (inner lane) to the outer (outer lane) as 31, 32, 33, etc. Lane numbering rules specify that upward lanes are listed first, followed by downward lanes, with inner lanes listed before outer lanes in the same direction. If the target is not in the lane, it is represented as 0xFF. Invalid values are represented as 0x00. |
| 2 | Queue length | US | 2 | Relative Y-axis distance to the stop line, resolution accuracy 0.1 m. |
| 3 | Queue head | US | 2 | Relative Y-axis distance to the stop line, resolution accuracy 0.1 m. |
| 4 | Queue tail | US | 2 | Relative Y-axis distance to the stop line, resolution accuracy 0.1 m. |
| 5 | Number of vehicles in queue | US | 2 | Vehicles |
| 6 | Whether it exceeds the limit | T | 1 | 1. Exceeded limit. 0. Not exceeded limit. |
| 7 | Whether it overflows | T | 1 | 1. Overflow. 0. No overflow. |
| 8 | Real-time vehicle spacing | US | 2 | Resolution: 0.1m |
| 9 | Number of vehicles online | US | 2 | Vehicles |
| 10 | Average speed | US | 2 | Resolution 0.1 km/h |
| 11 | 85% speed | US | 2 | Resolution 0.1 km/h |
| 12 | Lead vehicle position | US | 2 | Relative Y-axis distance to the stop line, resolution accuracy 0.1 m, invalid value 0xFFFF. |
| 13 | Lead vehicle speed | US | 2 | Resolution 0.1 km/h |
| 14 | Trailing vehicle position | US | 2 | Relative Y-axis distance to the stop line, resolution accuracy 0.1 m, invalid value 0xFFFF. |
| 15 | Trailing vehicle speed | US | 2 | Resolution 0.1 km/h |
| 16 | Space occupancy rate | US | 2 | Percentage, resolution 0.1 |
| 17 | Reserved | US | 2 |  |

2.2.6 Real-time passing vehicle data (0x05: PassData)

| NO. | Name | Type | Bytes | Description |
| --- | --- | --- | --- | --- |
| 1 | Lane NO. | US | 2 | Road code rules for lanes: For upward traffic (toward the radar), lanes are numbered consecutively from the inner (inner lane) to the outer (outer lane) as 11, 12, 13, etc. For downward traffic (away from the radar), lanes are also numbered consecutively from the inner (inner lane) to the outer (outer lane) as 31, 32, 33, etc. Lane numbering rules specify that upward lanes are listed first, followed by downward lanes, with inner lanes listed before outer lanes in the same direction. If the target is not in the lane, it is represented as 0xFF. Invalid values are represented as 0x00. |
| 2 | Cross-section position | US | 2 | Relative Y-axis distance to the stop line, resolution accuracy 0.1 m. |
| 3 | Real-time cross-section vehicle speed | US | 2 | Resolution 0.1 km/h |
| 4 | Cross-section headway time | US | 2 | Seconds |
| 5 | Passing time | US | 8 | Reference 1.3.4 data timestamp definition. |
| 6 | Occupancy duration | US | 2 | Seconds |
| 7 | Cross-section occupancy status | S | 1 | 1. Entering. 0. Exiting. |
| 8 | Target Type | S | 1 | 0X00—other 0X01—bicycle 0X02—motorcycle 0X03—tricycle 0X04—bus 0X05—van 0X06—car 0X07—SUV 0X08—large truck 0X09—medium truck 0X0A—light truck 0X0B—dangerous goods cars 0X0C—engineering vehicles 0X0D—person 0X0E—medium bus |
| 9 | Reserved | T | 3 |  |